— Last month, in Part IV of our Harley-Davidson Night Train project, we mounted up every set of exhaust pipes we could lay our hands on in a frantic search for more power.

The winner was the beautifully finished Bassani Pro Street pipe which put out 101hp, was easy to install and was an excellent fit on our modified Night Train.

This time we look at eight sets of two into one exhaust sets for our Night Train project bike. Once again we installed each set and tested them on the dyno to get the hp figures. Then we made notes on our first hand impression of each set with respect to fit, finish and ease of installation.

Below is the list of two into one pipes we checked out, see it all tonight starting at 7:30pm Eastern, ONLY On Speedvision!

Bassani Super Street

Idle 79-80 db /3000 rpm 94-95 db

STD HP 98.4 T-99.0

Finish very good, ease of installation is moderate to tricky for first timers.

Thunderheader

Idle 81db / 3000 rpm 97db

STD hp 99.2 T-100.7

Flat Black finish is raw pipes were easy to install

Mid-USA 2-1

Idle 83 / 3000 rpm 97-98db

STD hp 96.8 T-99.1

Not too easy to install, fit and finish was very good but not that attractive

Supertrapp.com - with 20 discs

Idle 76-77db / 3000 rpm 92db

STD hp 96.5 T  97.9

Easy to install and good lookin too

Supertrapp.com - with 15 discs

hp 93.5 T 98.7

Chrome Specialties 2-1

Idle 73-74 db 3000 88-89db (quiet)

HP 81 & 96.5 (no brackets or clamps)

White Brothers e-pipe (12 discs )

Idle 78-9 db 3000 db 94-96

HP97.6 hp and 95.8 t

Mounting shields took time fit a little rough but finish is good (nice wide torque band)

Hooker 2-1 Tuned Flow

Idle 75-76db 3000 rpm 93-95db

HP 78.0 T-96.7 torque tuned (closed)

OPENED up to HP 91.3 T-98.8**

easy installation

Hard Krome 2-1 (Double D)

Idle 79-80 db 3000 rpm 94-96db

HP 91.8 T-97.8

Dippy torque curve with low bass note, fit is good, but clamps were not useful.

Below is the list of pipes we put through the mill in Part IV, just in case you missed it.

Bassani Pro Street

Idle 81-83 db / 3000 rpm 96-98db

STD HP 101hp T- 97

Finish very good ease of installation is good

D and D

Idle 81-82 db / 3000 rpm 96db

STD HP 89.1 T- 98.3

Finish is very good easy to install

Squeaky sound

Cycle Shack

Idle 80db / 3000 rpm 97-98db

STD hp 98.8 T- 98.8

Black Chrome finish is cool and pipes were easy to install

Very Impressive, wide and flat torque curve

Bartels Performance

Idle 78-80 db /3000 rpm 94-95 db

STD hp 97.8 T 99.0

Possibly the best looking pipes.

Installation with heat shields moderate

Big sound and high power

Paucho dual tapered 38"

Idle 75 db / 3000 rpm 92-93 db

STD hp 82.7 T- 93.00

Easy to install** fit and finish good

Paucho Straight 40"

Idle 83-84 db / 3000 rpm 98-99 db

STD hp 97.2 T- 100 (no bottom end)

Easy to install** fit and finish good

High Peak Torque*

BUB 2" CONE DUALS

Idle 79-81db 3000 rpm 94-95db

HP 95.1 T- 96.8

Nice looking good fit and finish

Samson Challenger

Idle squeaky 79-81 db 3000 rpm 97-8 db

HP 91.5 T-97.2 (assemble required)

Installation is a little complicated

Nice choice of baffles and packing

SAMPSON BIG GUN II cannons

Idle 78-9 db 3000 rpm 95-96db

HP 80.6 T- 94.2 ( 18 gauge steel )

Hard Krome 2-2

Idle 77 db 3000 rpm 94-96 db

HP 89.2 T- 95.5

Easy install